Lock / Tag / Try Procedures

Purpose:	To prevent the accidental release of energy to equipment being cleaned,

maintained or repaired

Planning:	Supervisor to lead determination of all sources of energy

a)  blueprints, manuals and operator input

b)  isolation points – upstream and downstream

Methods of lockout:	Supervisor shall determine proper lockout device to render equipment inoperable.

a)  Locks (if used):

-supervisor

1)  uniquely keyed

2)  first on / last off

- employees

1)  identifiable – individual / department

2)  removable by supervisor’s lock

- Tags:

- individual / department

- date / time

c)  Other

Notify work area employees as to affected area and approximate time.

Try:             	Physically test isolation points after Lockout / Tagout to assure equipment

is inoperable.

Return to service:  Reverse Order of LTT shall be followed.

1.  Locks, Tags and other devices removed “first on, last off”

2.  Energy restored in reverse order of “shut down”

Five questions Supervisor must be able to answer:

- When is LTT used?

- What type of energy is to be LTT?

- Where are the isolation points?

- Who does the LTT?

- How will LTT be accomplished?

*Contractor responsibilities:  falls under supervisor of area affected

9/04